INTERNAL STRATEGY MEMO · VERSION 0.1 · 27 JULY 2026

Business model: batteries as dispatchable participants in India’s P2P energy markets

Working thesis, customer model, economics, beachhead and validation plan for kWh Electric.

RECOMMENDATION Build the OEM-neutral battery participation layer beneath trading apps and DISCOM programs. Do not launch as a standalone electricity exchange, token or consumer wallet.

# The thesis in one paragraph

India has proven P2P solar trading in Lucknow, Delhi and Kolkata and is now operating an interstate IES pilot with multiple trading applications. The application layer is becoming interoperable; the physical-asset layer is still fragmented. kWh Electric connects heterogeneous batteries, establishes safe and renewable-eligible availability, executes market schedules at the edge, and produces the evidence that trading providers and DISCOMs need for settlement. The business earns from enabling and operating the asset—not from owning the electricity trade.

# Strategic choices

| Choose | Avoid | Reason |
| --- | --- | --- |
| OEM-neutral control layer | Twelfth trading app | PVVNL already lists eleven authorised providers; physical integration remains fragmented. [3] |
| Delhi solar + storage beachhead | Nationwide launch | Delhi expressly includes renewable-charged BESS; permissions remain state-specific. [1] |
| Blended hardware + SaaS + enterprise revenue | P2P fee-only model | ₹0.42/kWh is too thin to fund device infrastructure. [2] |
| P2P as one service in a value stack | Battery built solely for energy resale | Backup, self-consumption and flexibility protect the customer value proposition. |

NON-NEGOTIABLE The owner reserve, OEM warranty, thermal limits and DISCOM authority outrank every market dispatch.

1 · MARKET EVIDENCE

From demonstrations to regulated settlement

| When / where | Pilot | Public evidence | Business implication |
| --- | --- | --- | --- |
| 2020 · Lucknow | UPPCL/MVVNL + ISGF + Powerledger | 12 participants; pilot recorded as successful; regulator asked for scale and storage. [6] | Pilot can create the rulebook. |
| 2021 · North Delhi | Tata Power-DDL + ISGF + Powerledger | More than 2 MW; ~150-site design; dynamic P2P logic. [7] | Large urban DISCOMs will test live cohorts. |
| 2022 · Kolkata | CESC + ISGF + Powerledger | 1,001 participants; 10% buyer-rate reduction reported. Units in public volume data conflict. [8] | DISCOM value must be part of the proposition. |
| 2026 · Delhi–UP | TPDDL + BRPL + PVVNL under IES | Interstate, multi-app pilot; 11 providers listed by PVVNL. [2][3][4] | Marketplace software is contestable; asset control is the wedge. |

# The operating model regulators have accepted

- The DISCOM verifies the participant and network, supplies meter data and remains responsible for the bill and consumer relationship.

- A service provider matches scheduled kWh at a mutually agreed price and reconciles schedule versus actual meter data.

- Physical power continues through the distribution network; P2P is a contractual, measurement and settlement overlay.

- Delhi includes a BESS only when it is charged through a renewable energy system; general grid-charge-and-resell arbitrage is not established. [1]

- The 2026 Delhi pilot permits a total transaction charge of ₹0.42/kWh inclusive of GST, split between buyer and seller, and rejects independent off-bill settlement. [2]

# Why the timing is real

| 40 lakh+ PM Surya Ghar beneficiary households by May 2026 [5] | 30.11 GW grid-connected rooftop solar by June 2026 [17] | 236.22 GWh projected BESS requirement in 2031–32 [16] |
| --- | --- | --- |

These are national context metrics, not kWh Electric's serviceable market. The initial SAM is the number of export-capable, smart-metered solar-plus-storage sites inside a participating DISCOM territory.

2 · PRODUCT AND CUSTOMER

The asset participation layer

CATEGORY A secure edge gateway, digital twin and policy engine that turns an OEM battery into a program-ready asset for any approved market application.

# Product boundary

| Layer | kWh Electric owns | kWh Electric does not own |
| --- | --- | --- |
| Physical | Gateway connection to BMS, inverter and site meter; local control | Battery cells, inverter hardware, DISCOM meter |
| Digital twin | Normalised capability, SoC/SoH, safe limits, permissions and owner reserve | OEM warranty policy; it is encoded and enforced |
| Market execution | Availability forecast, schedule-to-dispatch, acknowledgements, audit log | Price matching, customer wallet, exchange licence |
| Settlement evidence | Device telemetry, renewable provenance, exception record and reconciliation | Final regulated meter reading and electricity bill |

# Who pays, and for what job

| Customer | Job to be done | Economic buyer | Initial offer |
| --- | --- | --- | --- |
| Battery OEM | Make each model P2P/IES-ready without utility-by-utility engineering | Product / platform head | Model integration + embedded gateway + fleet API |
| Trading provider / aggregator | Control mixed OEM fleets through one interface | Founder / market operations | Portfolio API + device orchestration |
| DISCOM | Obtain safe, auditable flexibility and improve schedule delivery | Innovation / DER / DSM team | Pilot, control assurance and fleet observability |
| Owner / EPC / financier | Preserve backup while monetising an installed asset | Owner or channel partner | Bundled gateway + recurring service |

# Core capabilities for v1

- Local CAN / RS-485 / Ethernet integration and reusable device profiles.

- Capability discovery: charge/discharge limits, power, energy, alarms and export ability.

- SoC/SoH and available-energy forecast with an explicit owner-reserved backup floor.

- Renewable-charge provenance ledger separated from the market ledger.

- Policy-controlled dispatch, local fallback and signed command/acknowledgement trail.

- Boundary-meter reconciliation and IES/trading-platform API adapter.

3 · COMMERCIAL MODEL

Sell enablement and orchestration—not electricity

| Revenue line | Payer | Planning range — hypothesis | Trigger and notes |
| --- | --- | --- | --- |
| Model integration / NRE | Battery OEM | ₹10–25 lakh per product family | Protocol, safety policy, certification evidence and pilot support; validate in interviews |
| Gateway / embedded licence | OEM, EPC or owner | Target ~₹6,000 per Model A unit | Existing product target at volume; BOM and gross margin not yet locked |
| Residential orchestration | OEM, aggregator or owner | ₹149–299 per active device / month | Telemetry, forecasts, control policy, audit and API |
| C&I orchestration | OEM, aggregator or asset owner | ₹2,000–10,000 per site / month | Depends on kW, telemetry rate, SLA and market stack |
| Enterprise platform minimum | TSP, OEM or DISCOM | ₹10–50 lakh / year | Portfolio API, program configuration, support and reporting |
| Performance share | Aggregator or owner | 10–20% of verified incremental value | Only where contract and regulation permit; never assume a share of the regulated transaction fee |

PRICING PRINCIPLE Charge for model enablement, active control and enterprise assurance. Treat market revenue share as upside, not the base case.

# Offer architecture

| Offer | Includes | Best first buyer | Success gate |
| --- | --- | --- | --- |
| OEM Launch | One model profile, lab integration, pilot devices, market-ready claim pack | Ola/Tata/Livguard-style product team | Safe read/write control on a shipping model |
| Fleet Connect | Gateway, digital twin, monitoring, policy and API | OEM, EPC, financier | ≥95% online / control acknowledgement target |
| Market Dispatch | Availability forecast, schedule execution, audit and reconciliation | Trading provider / aggregator | Under-injection improvement and net owner value |
| DISCOM Assurance | Fleet visibility, permissions, exception handling and program reports | DISCOM DER/DSM team | Accepted data and controls for a live program |

# Channel logic

- OEM embed is the compounding route: one device profile unlocks every shipped unit of that model.

- Trading providers accelerate market access but should not own the kWh device relationship or data model.

- DISCOM approval creates trust and program eligibility; it is a channel and control authority, not necessarily the first SaaS payer.

- EPCs and financiers are later channels once the OEM and DISCOM acceptance path is repeatable.

4 · ECONOMICS

P2P alone does not pay for the battery—or the platform

The correct question is not whether P2P can finance a new battery. It is whether kWh Electric can add profitable incremental revenue and grid value to a battery already purchased for backup, self-consumption or resilience.

# Illustrative residential throughput

| Input | Illustrative assumption | Result / interpretation |
| --- | --- | --- |
| Nominal battery | 9.1 kWh | Comparable to the largest public Ola Shakti configuration [9] |
| Usable depth of discharge | 90% | 8.19 kWh available before conversion [9] |
| Dispatch efficiency | 90% | 7.37 kWh delivered per full market cycle |
| Market days | 300 per year | 2,211 kWh delivered annually |
| Incremental P2P value | ₹2.00 per delivered kWh | ₹4,422 gross annual owner value before degradation |
| Regulated platform fee pool | ₹0.42 per delivered kWh | ₹929 per year across buyer + seller—not kWh revenue [2] |

Illustrative assumptions are not a forecast. Actual value depends on tariff, feed-in compensation, solar surplus, loss factors, reserve, cycles, warranty and market rules.

# Degradation is the economic gate

FORMULA Degradation cost per delivered kWh = battery-attributable capital ÷ lifetime delivered kWh. The market spread must exceed this cost—or the dispatch must earn another value stream.

| Illustrative battery capital | Cycle-life assumption | Lifetime delivered energy | Degradation cost |
| --- | --- | --- | --- |
| ₹1.50 lakh | 4,000 full-equivalent cycles | ~26,536 kWh | ~₹5.65 / delivered kWh |
| ₹2.50 lakh | 4,000 full-equivalent cycles | ~26,536 kWh | ~₹9.42 / delivered kWh |

The ₹1.50 lakh and ₹2.50 lakh cases approximate Ola Shakti's launch and current published 9.1 kWh price points; 4,000 cycles and 90% efficiency are internal scenario assumptions, not Ola specifications. [9][10]

# kWh recurring-revenue sensitivity

| Active devices | ₹199 / device / month | ₹299 / device / month | Meaning |
| --- | --- | --- | --- |
| 1,000 | ₹23.9 lakh ARR | ₹35.9 lakh ARR | Pilot-to-product stage |
| 10,000 | ₹2.39 crore ARR | ₹3.59 crore ARR | One scaled OEM or multi-channel fleet |
| 100,000 | ₹23.88 crore ARR | ₹35.88 crore ARR | National embedded distribution |

ECONOMIC CONCLUSION Use residential P2P to prove interoperability and OEM distribution. Seek the first material revenue in C&I/portfolio orchestration, where throughput, demand charges and grid value are larger.

5 · BEACHHEAD AND GO-TO-MARKET

Win Delhi; distribute through the OEM

# Beachhead

| Dimension | Choice | Why |
| --- | --- | --- |
| Regulatory territory | TPDDL or BRPL, Delhi | Renewable-charged BESS is explicit; 2026 P2P pilot orders are active. [1][2] |
| Asset class | Solar + 5–40 kWh storage | Matches emerging residential/SMB portfolios and Delhi's ≤200 kW participant ceiling |
| Commercial partner | One authorised IES TSP | Avoid building market matching and billing workflows already supplied by others. [3] |
| OEM partner | One open, motivated battery manufacturer | A device profile creates a repeatable distribution path |
| Buyer pool | Residential + small commercial consumers | Diverse load shapes create a better demonstration than households alone |

# 90-day pilot design

- Days 0–30 — integrate one OEM model in lab; establish device profile, safe controls, permissions and renewable-energy accounting.

- Days 31–60 — install 25–50 sites; connect TSP and DISCOM interfaces; establish solar-only baseline and owner reserve policy.

- Days 61–90 — submit day-ahead availability, execute accepted schedules, reconcile meter data and produce a regulator-ready result pack.

# Pilot scorecard

| Metric | Target / method | Why it matters |
| --- | --- | --- |
| Integration effort | One reusable profile per model; no site-specific driver | Proves compounding software economics |
| Availability forecast | Error tracked by time block and site | Determines offer confidence |
| Schedule fulfilment | Battery cohort vs solar-only baseline | Primary P2P value proof |
| Control reliability | ≥95% acknowledged commands; exceptions explained | DISCOM and OEM trust |
| Reconciliation | Target <2% gateway-to-boundary-meter variance | Settlement credibility |
| Net owner value | Revenue/savings less losses, fees and degradation | Prevents false-positive economics |
| Safety / autonomy | Zero reserve, warranty or thermal violations | Licence to operate |

# Sales sequence

1 · OEM technical sponsor → 2 · authorised TSP integration → 3 · DISCOM pilot approval → 4 · embedded commercial agreement → 5 · second OEM / second program

6 · OEM AND PARTNER LANDSCAPE

Targets are product ecosystems, not just cell companies

“Tata cells” resolves into multiple relevant entities. The near-term product partner is Tata Power / TPREL's MySine and Battery Storage portfolio; Agratas is a cell-manufacturing platform and is not the immediate route to a P2P-ready end product.

| Target | Public product signal | Public control signal | kWh opportunity | Priority |
| --- | --- | --- | --- | --- |
| Ola Shakti | 1.5, 5.2 and 9.1 kWh public configurations; 90% DoD; NMC 4680 cells [9][10] | Wi‑Fi, 4G and app; no public market-control API found | Add OEM-grade local interface, safe export and IES participation | High — clear mass-market product, interface diligence required |
| Tata Power / TPREL | MySine LFP; 6,000+ cycles claimed; new 5–40 kWh residential and 60 kWh–5 MWh PowerHub families [11][12] | Mobile monitoring; Tata also reports BESS observability and a P2P platform | Partner on device standardisation or fill interoperability gaps outside the closed Tata stack | High strategic value, harder channel conflict |
| Amara Raja | 1 GWh lithium storage across ~50,000 telecom sites [13] | Large distributed operational fleet; interfaces not public | C&I/telecom fleet control and program participation | High for scale; not residential-first |
| Livguard | Lithium X home hybrid ESS; remote monitoring and smart BMS [15] | Remote monitoring advertised; market API not public | Faster residential integration and channel test | Medium-high |
| Exide | Large solar and residential backup channel; current public solar portfolio remains lead-acid heavy [14] | No public dispatch API found | Retrofit monitoring first; controlled cycling only where warranty supports it | Medium; huge channel, weaker initial cycling fit |

# Partner selection gates

- Local read/write interface documented (CAN, Modbus, SunSpec or supported gateway SDK).

- Grid-parallel export technically supported by the inverter and installation design.

- Warranty permits controlled cycling and exposes enough telemetry to enforce limits.

- OEM will allow signed third-party commands under explicit owner consent.

- A product leader has a reason to launch a market-ready differentiation within 6–9 months.

DILIGENCE RULE App control is not equivalent to a safe market API. No external claim of compatibility until read/write control, export mode, warranty and certifications are verified on the exact SKU.

7 · TECHNICAL AND OPERATING MODEL

Control at the edge; authority stays explicit

# Control hierarchy

| Priority | Authority | Rule |
| --- | --- | --- |
| 1 | Electrical protection / BMS | Hard safety trip always wins |
| 2 | Owner reserve and opt-out | Backup floor and manual override cannot be traded away |
| 3 | OEM warranty policy | Temperature, current, power and cycle constraints enforced locally |
| 4 | DISCOM emergency / network instruction | Curtail or disconnect within approved program rules |
| 5 | Accepted P2P / flexibility schedule | Execute only inside the remaining safe envelope |
| 6 | Self-consumption / tariff optimisation | Optimise residual flexibility |

# Minimum data contract

| Read | Write | Derived | Audit |
| --- | --- | --- | --- |
| Power, energy, SoC, SoH, voltage, current, temperature, alarms, connectivity | Charge/discharge enable, active-power limit, mode, reserve, ramp rate | Safe available kWh, renewable-attributed kWh, forecast, degradation proxy | Command, policy version, consent, acknowledgement, exception, meter variance |

# Permission model

- Owner grants purpose-limited access to named services and can revoke it.

- The trading provider receives market availability and execution status—not unrestricted BMS control.

- The DISCOM receives verified meter-linked performance and emergency authority defined by the program.

- The OEM retains warranty policies and device-level safety authority.

- kWh records the policy decision and local outcome for every command.

# What must work without cloud connectivity

EDGE FALLBACK Hold owner reserve, reject unsafe commands, complete or safely abort the active interval, buffer telemetry, and reconcile when connectivity returns.

# IES alignment

Treat IES as the northbound trust and interoperability layer: verified participant/asset identity, consented data exchange, standard payloads and audit. Do not make IES—or any blockchain—the device safety controller. [4]

8 · 24-MONTH EXECUTION PLAN

Prove one model, then make the library compound

| Phase | Months | Deliverables | Commercial gate | North-star KPI |
| --- | --- | --- | --- | --- |
| 0 · Partner lock | 0–2 | OEM technical sponsor, TSP partner, Delhi DISCOM path, exact SKU | Signed pilot MoU / paid discovery | One writable battery SKU |
| 1 · Integration | 2–5 | Device profile, policy engine, provenance, lab validation, pilot gateway | OEM acceptance test | Safe dispatch success |
| 2 · Live pilot | 5–8 | 25–50 sites, baseline, schedules, reconciliation, result report | Pilot converts to commercial programme | Schedule fulfilment uplift |
| 3 · Productise | 8–14 | Installer tooling, fleet API, SLA, billing, second SKU | Per-device recurring contract | Active paid devices |
| 4 · Expand | 14–24 | Second OEM, C&I offer, second DISCOM/state, additional market service | Portfolio/enterprise renewal | MW under management |

# Team and spend priorities

| Workstream | First capability | Why first |
| --- | --- | --- |
| Embedded / protocols | CAN, Modbus, inverter/BMS profile tooling | Unlocks the asset |
| Controls / forecasting | Safe envelope, reserve and available-energy forecast | Determines reliable offers |
| Utility integration | IES/TSP adapter and meter reconciliation | Unlocks settlement |
| Field operations | Installer workflow, monitoring and exception playbook | Converts lab reliability into fleet reliability |
| Regulatory / partnerships | DISCOM and OEM responsibility matrix | Prevents a technically valid but unlaunchable product |

# KPI tree

North star: dispatchable MW under management  →  active paid devices  →  safe schedule-delivery rate  →  number of reusable device profiles  →  net value delivered per active asset.

9 · RISKS, DECISIONS AND VALIDATION

What can break the thesis

| Risk | Failure mode | Mitigation / test | Kill criterion |
| --- | --- | --- | --- |
| Battery economics | Spread is below degradation and loss cost | Model net incremental value by tariff and SKU; stack other services | No positive use case after backup reserve and degradation |
| OEM access | Closed API or warranty blocks external control | Start with local protocol and signed policy; target open sponsor | No production read/write path on exact SKU |
| Regulatory scope | Battery not eligible or only solar energy may be traded | Track energy provenance; launch in Delhi; keep P2P one app | No regulator/DISCOM path to a live battery dispatch |
| DISCOM value | Utility sees revenue erosion or operational risk | Measure peak, schedule and local-grid value; give DISCOM control | Pilot cannot name a utility-side economic benefit |
| TSP commoditisation | Trading provider bundles device control | Remain multi-platform and build deep OEM/profile library | TSP controls OEM interface and makes kWh redundant |
| Safety / cyber | Unsafe or unauthorised dispatch | Edge hierarchy, signed commands, least privilege, immutable audit | Any unexplained reserve, thermal or permission violation |

# Founder decisions required

- Choose the first commercial focus: residential OEM distribution or C&I portfolio value. Recommendation: residential for proof, C&I for first material revenue.

- Confirm whether the ~₹6,000 Model A price is hardware only, or hardware plus software activation and connectivity.

- Choose kWh's control-authority stance: direct dispatch under OEM permission, or recommendation-only for the first pilot.

- Select the first target OEM based on interface openness—not brand size.

- Decide whether kWh will register as a trading service provider later. Recommendation: remain infrastructure-neutral through the first two pilots.

# Next 30 days

| Action | Output | Owner / proof |
| --- | --- | --- |
| OEM diligence | Exact SKU interface, export mode, warranty and cycle policy for Ola Shakti, MySine/TP Battery Storage and one open alternative | Three technical calls + written interface evidence |
| TSP diligence | Select one authorised provider and map its schedule, API and settlement contract | Sandbox or API session |
| DISCOM diligence | Confirm battery eligibility, meter interval, baseline and pilot approval route | Written pilot responsibility matrix |
| Value model | Tariff × solar × battery × degradation scenario calculator | Three viable site archetypes |
| Pilot pack | Architecture, SOW, safety case, data contract and scorecard | Partner-ready proposal |

GO / NO-GO GATE Proceed to product build only when one exact battery SKU is writable, one authorised market partner will consume the API, and one DISCOM confirms a live settlement path.

APPENDIX · EVIDENCE AND ASSUMPTIONS

Source register

This is a working business model, not legal, regulatory or investment advice. Planning ranges are hypotheses for partner discovery. Public pilot metrics have been conservatively labelled where source units conflict.

## Primary and analytical sources

[1] DERC Peer-to-Peer Energy Transaction Guidelines, 2024

[2] DERC Order in Petition No. 02/2026 — TPDDL P2P pilot

[3] PVVNL — current IES P2P pilot and authorised trading applications

[4] REC — India Energy Stack P2P Trading EOI

[5] MNRE/PIB — PM Surya Ghar crossed 40 lakh beneficiary households

[6] UPERC — P2P Solar Energy Transaction Guidelines

[7] Tata Power-DDL — live P2P pilot announcement

[8] IEEFA/JMK — rooftop solar market and CESC P2P case study

[9] Ola Electric — Ola Shakti specifications

[10] Ola Electric — Shakti product-launch disclosure

[11] Tata Power — MySine Energy Storage System

[12] Tata Power — battery-storage portfolio launch

[13] Amara Raja — 1 GWh lithium storage deployed across telecom sites

[14] Exide Industries — solar battery portfolio

[15] Livguard — Lithium X hybrid residential energy storage

[16] Ministry of Power — National Framework for Promoting Energy Storage Systems

[17] MNRE — renewable-energy physical progress to 30 June 2026

## Research companion

Full pilot chronology, regulatory mechanics, caveats and recommended beachhead: research/p2p-trading-india-pilots.md